SJCam M10+ Gyro Firmware update V1.5-v1.8 and later 
Upgrading Tutorial

Note: if you are upgrading from a version 1.4 or lower, you will need to also install the loader file. For example, you cannot just upgrade from v1.3 to v1.8 without also installing the loader file.

Youtube Tutorial is here: https://www.youtube.com/watch?v=y4rAE7-ugkw

The SJCAM M10+ 2K Gyro it's receiving a newer firmware that adds bug fixes and improvements. This new update will also need a one time additional step to prepare the camera for future updates.

Please pay attention to the following official update procedure

Updating firmware Version 1.0 for the SJCAM M10+ plus has several new steps. You will need to install a Loader file first (LD96660A.bin) and then the Firmware (SJCAMM10.bin). The firmware and process is only for units with M10+ v1.7 firmware and below. Firmware versions above 1.7 will not require the first step which is installing the Loader software.

Warning: Once the M10+ firmware has been updated to Version 1.7, you cannot flash the M10+ to firmware versions below v1.5. This firmware is only for units with Versions v1.4 and above. If you revert back to a firmware version lower than v1.5 (like v1.3, v1.4), you will brick your camera and it will no longer be functional (black screen).

Installing Loader Software:
a)Make sure to format the microSD card.
b)Copy the loader “LD96660A.bin” into the root of your MicroSD card.
c)Insert microSD card into the SJ4000+ unit and power the camera on. Note: makes sure the camera is off when inserting the card. Once the camera is powered on, turn the M10+ off and remove the card. 
d)Delete the loader LD96660A.bin from the MicroSD card. Next, copy v2.6 firmware file “SJCAM10.bin” into the root of the microSD. Note: there is no need to format the card again
e)Re-insert the MicroSD into the powered off M10+ and power the unit on. It may take 10-15 seconds for the firmware to install. The blue and yellow light will blink until the firmware is installed and then the unit will power on. Check the firmware version to confirm version has successfully been updated then delete the firmware file SJCAMM10.bin from card otherwise the firmware will re-install upon re-powering the unit on.

V1.7 Official Changelog

1) Fixes when low illumination and changing modes, the LCD will flicker.

2)Improves on White Balance

3)Improves on color – adds color options and values

4)Fixes after closing the Time-stamp, shooting 720p @120FPS & VGA 240FPS unit will crash

issue.

5)Fixes after turning GYRO On, changing modes the LCD screen shakes

6)Fixes when shooting 720p@120FPS Gyro is not functional; Please note: VGA 240FPS will not

support Gyro

7) Adds Volume adjustment menu for video

8) Adds Photo quality menu options

9) Adds Sharpness menu options

10) Adds Color options

11) Adds FOV options

12) Adds Underwater op

13) Adds file format option: Record in .MOV or MP4

14) Adds optional SJCAM logo option

15) Improves menu items order

1.6 Official Changelog
1) Fixes the Duplicate License Plate Number watermark issue under "LicenseNoSet" menu option. Also fixes the license mark not being removed when option is deselected.
2) Fixes issue when after the unit has been powered off, then powered back on again, the Gyro settings need to be reset. Adds a Gyro Sensor Quick Key - when using SJ4000+ to record or take pictures, gently press the WiFi button to switch the Gyro off/on. Press the button down longer for Zoom.
3) Improves automatic white balance (AWB)
4) Fixes left side of the screen has a line issue.
5) Enhances Wifi signal strength and improves Wifi signal stability.


Warning: Once the M10+ firmware has been updated to Version 1.7, Do Not flash the M10+ to lower firmware versions.  If you revert back to a firmware version lower than v1.5 (like v1.2, v1.3, v1.4) you will brick your unit and camera will have a black screen.